# Nets Pay@Table Setup Guide v1.1

Version: 1.1
Date: 11.07.2013

Table of Contents

1. Introduction	3

1.1 System overview	3

1.2 Mobile Manager	4

2. Setup	5

2.1 Installation of Generic MTMSServer	5

2.1.1 Demo POS Server	5

2.2 Terminal setup	5

2.2.1 Set server IP	5

3. Trace file	7

4. Reinstallation of Mobile Manager	9

5. Common issues and questions	10

5.1 How to do a refund/reversal with “Demo PosServer”	10

5.2 How to go to normal payment mode when Dialogue-app is running	10

5.3 How to open the Dialogue-app when in normal payment mode	11

# 1. Introduction

This document is intented for Pay@Table integrators and serves as a setupguide for the payment terminal and the MTMSServer. The document is a working progress and will be updated rapidly depending on feedback and questions from it’s readers.

Note: To follow the instructions in this document it is expected that the payment terminal is already configured with a valid terminal ID(TID) which is set up for P@T.

### 1.1 System overview

POS Server

Nets Mobile Manager

TCP/IP

Server

Client

Bluetooth setup

Generic

Mobile Manager API

Nets Dialogue API

Key:

Subsystem

SW/HW component 
(restaurant / system provider) /system integrator / others

Black

Blue

Network technology

SW Protocol

POS terminal n

POS terminal 1

Nets ECR protocol

SW/HW component (Nets)

Pay@Table
Terminal

ITU

Cradle

Bluetooth

Nets

PSP/
Baxbis

LAN / WAN

Nets host protocol

L
AN

TCP/IP

IP server accessed by payment terminal

### 1.2 Mobile Manager

# 2. Setup

### 2.1 Installation of Generic MTMSServer

The first step to make Pay@Table possible is to install the MTMSServer, which is included in Mobile Manager. MTMSServer is a windows service which must always be in started state while using P@T features.

Install:

- Unzip the MTMSServer zip-file

- Copy the MTMSServer folder to “C:\Manison”

- Run “C:\Manison\MTMSServer\MTMSServiceInstall.cmd

“MTMSServer” should now be listed under “services.msc”. Start the service if isn’t already.

Before a successful connection can be made between the terminal and the POS server, the server must accept incoming TCP-connections on specified port (default is 7779). Because port-forwarding depends greatly on type of firewall, router, OS etc. it will not be covered by this document.

2.1.1 Demo POS Server
The generic package also includes a demo POS Server which is useful for testing the connection between Terminal/Mobile Manager/POS Server as well as basic functionality.
To start the demo POS, run “C:\Manison\MTMSServer\DemoPosServer\DemoPosServer.exe”. The demo POS server contains several checks available for payment. When all checks are paid, it is necessary to restart the demo POS to get a new list of checks.

It is possible to change the Locked and Open status in the DemoPos. To toggle between Locked=False and Locked=True, press the F2 button while the check is marked. The same can be done with Open=False and Open=True by using the F1 key.

### 2.2 Terminal setup

When the Pay@Table terminal boots up, it will automatically load the “Dialogue”-application. The UI provided to the terminal under this application is received from Mobile Manager.

Before the terminal is able to connect to the POS Server, the server IP must be defined in the terminal.

### 2.2.1 Set server IP

- Press the “Menu”-button to get a list of available applications

- Select “TcpTunnel”

- Enter the “Settings”-menu

- Select “URL”

- Type in the server IP and confirm with Ok

To test the connection, return to the “Dialogue” application by pressing “Stop” followed by “Menu”. Go to “Dialogue” again and select “CONNECT”. Make sure that MTMSServer is running and that the cradle is connected to the same network as the server. If no errors are shown, the terminal now has a successful connection to the server.

# 3. Trace file

Mobile manager generates a trace-file containing useful information for debugging and troubleshooting. The trace file is located at “C:\Manison\MTMSServer\Trace_yyyyMMdd.txt”.

The trace file contains:

- All interface messages towards POS

- Receipt data

- Errors

  - Script Errors

  - POS returned errors

  - Terminal returned errors

- Statistics

  - Number of approved/rejected transactions

  - POS response times

  - Data communication statistics

  - Terminal information

- Debug information

  - Interface messages towards terminal

  - Data communication debug

  - Script debug

<Message ID="Checks" Direction="Response" Sender="RmsV5 /60 Test1 ">

<Status Result="0"/><Info Terminal="76565701" SequenceRef="30"/>

<Data>

<Check ChkID="T000010001" ReceiptNro="0" Amount="15600" TransactionType="0" Text="T:1 0"

</Data>

</Message>

In the below example there are two syntax errors in the XML-response from the POS server:

09.01.2013 11:06:55.759 AppPosServer.20701: APPLICATION ERROR

Number       5

Description  Name cannot begin with the '<' character, hexadecimal value 0x3C. Line 2, position 228.

Source       System.Xml

Function     AppPosServer.Application.Command_GetChecks

Erl          0

LastDLLError 0

09.01.2013 11:06:55.759 AppTermManager(76565701): ProcessInternalCommand: Response status: 10=Failed Group/Step: 1/30

09.01.2013 11:06:55.759 AppTermManager(76565701): WaitData: Error no steps to process: EntryGroup/EntryStep: 5/20

The trace file states what went wrong and where:

From the logged error, you can see that there was something wrong at Line 2, position 228(the example is formatted for better readability, so the position in this case is inaccurate). Here there was no end tag to the <Check>-node. The second error is that the “ChkID”-attribute contains alphanumerical characters.

“Group/Step” refers to the “<StepGroup>” and “<Step>” tags in the script file respectively.

# 4. Reinstallation of Mobile Manager

To re-install Mobile Manager, replace the old package(“X:\Manison\MTMSServer”) with the new one. Before replacing, take a backup of the current package (in case of problems with the new version).

The following files can be transferred from the old package so parameter-changes done on the terminal don’t have to be made again:

- “MTMSServer\ApplicationData_20700\Terminal.xml”

  - Terminal specific configurations (RVC, service mode, receipt prompt)

- “MTMSServer\ApplicationData_20700\LunchCouponItems”

  - Configured lunch coupons

# 5. Common issues and questions

5.1 How to do a refund/reversal with “Demo PosServer”
Select an arbitrary check for payment and note down the receipt number. After payment is complete, the “Demo PosServer” will look similar to this:

The check (receipt number 87) will be closed. To do a return select the check and press DELETE(on keyboard). A new check of type REFUND will be created:

“Receipt 94” will now be listed on the terminal. Select the check and enter the original receipt number when prompted. Confirm the return.

5.2 How to go to normal payment mode when Dialogue-app is running
While in the “Enter cashier ID”-screen, enter “000000” and press “Ok”. Select “Admin Mode” to exit “Dialogue” and return the normal payment-application.

5.3 How to open the Dialogue-app when in normal payment mode
Press “F1” (upper left button) and select Dialogue-app.

### 5.4 How to associate the terminal with a new base station/cradle

While in the Dialoge-app, press the menu button and select “TELIUM MANAGER”. The password is 4302. Thereafter, press 3 -> 2 -> 4 -> 1 -> 1. The terminal will now prompt you to put it on the cradle which it is to be associated with.